SYSTEM ARCHITECTURE Mold-Part Defeaturing Team CleanCAD Junoh Lee (Team Lead) ¦ Hemanth Reddy Vangala ¦ Aneela Tahir ¦ Balcha Kidus Elias System Architecture - 5 October 2026 Scope note: This document describes the system-level architecture only. Detailed module design, pretrained-model internals, graph schemas, decision rules, data structures, APIs, UI screens, and function-level behavior are deferred to the HLD and LLD..

# Revised History

| Name | Date | Reason For Changes | Version |
| --- | --- | --- | --- |
| Hemanth Reddy Vangala | 2026-09-17 | Initial version (system-level pipeline diagram). | 1.0 |
| Hemanth Reddy Vangala | 2026-10-05 | Updated to the module-based architecture: nine modules, labeled interfaces, face mapping, visualization and file management modules. | 1.1 |
| Junoh Lee | 2026-10-08 | Added this revision history per professor's review. | 1.2 |

# 1. Overview

CleanCAD processes a STEP CAD model through CAD ingestion, B-rep extraction, graph conversion, pretrained feature recognition and classification, rule-based defeaturing decisions, CAD-kernel defeaturing, geometry healing, visualization, and file management. The final output is a healed defeatured STEP model together with removal metadata and a before/after visualization.

CleanCAD does not require a formal engineer approval step in the core runtime. After the system produces the defeatured STEP model, an engineer may perform additional manual defeaturing if further simplification is desired; that work is outside the CleanCAD system boundary.

## 1.1 Architecture Goals

Separate CAD geometry processing, graph conversion, pretrained feature recognition, decision logic, visualization, and file management into clear system modules.

Maintain traceable face correspondence from the original B-rep through graph conversion, feature recognition, removal decisions, and CAD defeaturing.

Use a pretrained feature-recognition/classification model at runtime so the prototype does not depend on a project-specific offline training pipeline.

Keep STEP as the supported CAD input/output format for the current project scope.

Store the original STEP file, final defeatured STEP file, and removed-feature metadata while keeping optional post-output manual defeaturing outside the CleanCAD system boundary.

# 2. Core System Architecture

Figure 1 shows the major CleanCAD runtime modules and the information exchanged between them. The architecture separates ingestion and graph conversion, feature recognition and classification, rule-based removal decisions, CAD defeaturing and healing, result visualization, and file management.

Figure 1. CleanCAD System Architecture

# 3. Major Components

| Component | System-Level Responsibility |
| --- | --- |
| CAD Model Ingestion Module | Accepts the original STEP CAD model and makes the input file available to downstream modules. |
| B-Rep Extraction Module | Extracts the model B-rep geometry and establishes the face mapping used throughout the runtime. |
| Graph Conversion Module | Transforms the B-rep into a face-adjacency graph with geometric attributes while preserving face correspondence. |
| Feature Recognition & Classification Module | Uses a pretrained feature-recognition model to identify feature types and associate them with original face IDs and relevant geometric properties. |
| Rule-Based Defeaturing Decision Module | Applies geometric removal rules to recognized features and selects the face IDs/features to remove. |
| CAD Kernel Defeaturing Module | Uses the original B-rep/face mapping and selected face IDs to remove targeted geometry, producing an intermediate defeatured B-rep. |
| CAD Healing & STEP Export Module | Repairs the intermediate defeatured B-rep into a valid watertight model and exports the final defeatured STEP file. |
| Result Visualization Module | Shows the original model with selected removal faces highlighted and the final defeatured model as the before/after result. |
| File Management Module | Stores the original STEP file, final defeatured STEP file, and removed-feature metadata in the project output location. |

# 4. Core Runtime Flow

A STEP CAD model enters the CleanCAD system through the CAD Model Ingestion Module, and the original file is made available to file management and visualization.

The B-Rep Extraction Module reads the STEP model and produces the B-rep geometry together with face mapping information.

The Graph Conversion Module converts the B-rep into a face-adjacency graph while preserving geometric attributes and the mapping back to original CAD faces.

The pretrained Feature Recognition & Classification Module analyzes the graph and returns recognized feature types, associated face IDs, and geometric properties.

The Rule-Based Defeaturing Decision Module applies removal rules to the recognized feature data and selects the face IDs/features to remove; the same decisions form the removed-feature metadata.

The CAD Kernel Defeaturing Module receives the original B-rep/face mapping and selected face IDs, removes the targeted geometry, and produces an intermediate defeatured B-rep.

The CAD Healing & STEP Export Module repairs the intermediate defeatured B-rep into a valid watertight model and exports the final defeatured STEP file.

The Result Visualization Module uses the original model, selected removal face IDs, and the final defeatured model to show the before/after result; File Management stores the original STEP, defeatured STEP, and removal metadata.

# 5. System Boundary and Scope

The CleanCAD runtime begins when a STEP file is provided to the CAD Model Ingestion Module.

The CleanCAD runtime produces a final healed defeatured STEP file, a before/after visualization, and removed-feature metadata.

Any additional manual defeaturing performed after CleanCAD output is optional and outside the core CleanCAD system.

Detailed graph schemas, pretrained-model internals, rule thresholds, face-mapping implementation, CAD-kernel API calls, metadata schemas, and UI behavior are deferred to the HLD and LLD.

# 6. Key System Interfaces

| Interface | Information Exchanged |
| --- | --- |
| CAD Model Ingestion -> B-Rep Extraction | Original STEP CAD file. |
| B-Rep Extraction -> Graph Conversion | B-rep geometry and face mapping needed to build the graph. |
| Graph Conversion -> Feature Recognition & Classification | Face-adjacency graph, geometric attributes, and mapping back to original CAD faces. |
| Feature Recognition & Classification -> Rule-Based Decision | Feature type, associated face IDs, and geometric properties used by the removal rules. |
| B-Rep Extraction -> CAD Kernel Defeaturing | Original B-rep geometry and face mapping so the defeaturing kernel acts on the same face partition. |
| Rule-Based Decision -> CAD Kernel Defeaturing | Selected face IDs / feature references to remove. |
| CAD Kernel Defeaturing -> CAD Healing & STEP Export | Intermediate defeatured B-rep requiring healing. |
| Decision + Original/Final CAD -> Result Visualization | Selected removal face IDs, original model, and final defeatured model for red-highlighted before/after display. |
| Ingestion / Decision / Healing -> File Management | Original STEP file, removed-feature metadata, and final defeatured STEP file. |

# 7. Architecture Constraints and Assumptions

| Constraint / Assumption | Architecture Implication |
| --- | --- |
| CAD format | STEP is the supported CAD input/output format for the current project scope. |
| Runtime ML model | Feature recognition/classification uses a pretrained model; the current prototype does not depend on a project-specific model-training pipeline. |
| Face correspondence | The mapping between B-rep faces, graph nodes, model predictions, and CAD face references must remain consistent through the runtime. |
| CAD processing | Defeaturing and healing operate on the internal B-rep representation; STEP export occurs after the healed geometry is produced. |
| Visualization | The CAD visualization layer uses the original model plus selected face IDs to highlight removal faces and displays the final defeatured model for comparison. |
| File storage | The prototype uses local file/folder storage for the original STEP, defeatured STEP, and removed-feature metadata; a separate database is not required. |
| Post-output work | Any additional engineer-driven defeaturing after CleanCAD output is optional and external to the core system. |
| Design depth | Detailed component internals and implementation choices are deferred to the HLD and LLD. |

# 8. Details Deferred to HLD and LLD

Specific pretrained feature-recognition model selection, architecture, and inference implementation.

Node/edge attributes, tensor formats, graph schemas, face-to-node mapping details, and serialization.

Exact rule thresholds and geometric criteria used to classify features as KEEP or REMOVE.

Detailed CAD-kernel defeaturing, geometry-healing, failure-handling, and STEP-export logic.

Class/function design, APIs, module schemas, logging, testing, and implementation details.

FreeCAD face-highlighting implementation, before/after display behavior, and user-interface screens.

Local folder structure, file naming conventions, and the detailed removed-feature metadata schema.