High-Level Design Document

Project Name: Mold-Part Defeaturing

Team Name: CleanCAD

Team members: Junoh Lee (Team Lead), Hemanth Reddy Vangala, Aneela Tahir, Balcha Kidus Elias

# Revised History

| Name | Date | Reason For Changes | Version |
| --- | --- | --- | --- |
| Junoh Lee | 2026-09-11 | Initial Version | 1.0 |
| Junoh Lee | 2026-09-17 | Removed premature pythonocc-core naming | 1.1 |
| Junoh Lee | 2026-09-18 | Toolchain confirmed as FreeCAD Python API; added deployment constraint (CPU inference), success criteria, staged Decision Logic, and failure-case retraining loop | 1.2 |
| Junoh Lee | 2026-09-29 | Added a note that the core mechanism (M1, M5, and the face-adjacency traversal M2 will build on) is already prototyped and verified in src/cleancad_core.py (9/9 example shapes). No architectural changes. | 1.3 |
| Junoh Lee | 2026-10-02 | Noted HP mentor feedback as open, unverified candidates: explicit Stage 1 dimension thresholds and a supervisory decision layer for rule/ML conflicts (M4), logo feature category (M3), separate tracking of removal-execution failures (M5). No architectural change decided. | 1.4 |
| Junoh Lee | 2026-10-02 | Per professor request: added FR/NFR requirement numbers for readability - new "Related Requirements" column in the module table, requirement IDs on the design constraints, and a requirements traceability matrix (Section 7). No architectural changes. | 1.5 |
| Junoh Lee | 2026-10-08 | Per professor's review: team name corrected to CleanCAD; NFR-USE-02 shown as Must in the module table and the traceability matrix. | 1.6 |

# 1. Purpose

This document describes the module-level (high-level) architecture of the Mold-Part Defeaturing system: the responsibility of each module, the interfaces between modules, and the technology stack. It elaborates on the Functional and Non-Functional Requirements defined in the FRD/NFRD, and will be refined into per-module Low-Level Design (LLD) in a later phase.

# 2. System Architecture Overview

The system consists of a core runtime pipeline (M1-M8) that processes one CAD model at a time, and a separate offline training pipeline that produces the trained feature-recognition model used by M3. The tool only proposes a defeatured result - every result is reviewed and approved by an engineer (M7) before being finalized.

Toolchain (confirmed 2026-09-18): FreeCAD's Python API, run headless as standalone batch/automated code. cadquery may optionally be used, but only for generating STEP files. Deployment constraint: inference must run on a GPU-less office laptop; only training may use a GPU server.

Figure 1. Core Runtime Pipeline (M1-M8)

Figure 2. Offline Model Training Pipeline

# 3. Module Descriptions

| Module | Responsibility | Technology | Related Requirements (W = Want) |
| --- | --- | --- | --- |
| M1 — CAD I/O | Parse input STEP file into a B-rep model; export the final result as a STEP file. | FreeCAD Python API, headless | FR-IO-01, FR-IO-02, FR-IO-03, FR-IO-04, NFR-REL-01, NFR-REL-03 (W), NFR-PORT-02, NFR-PORT-03, NFR-USE-02 |
| M2 — Graph Conversion | Convert the B-rep model into a face adjacency graph with UV parameter samples, usable as ML model input. Shared by both the runtime pipeline and the offline training pipeline. | occwl or a FreeCAD-API-based equivalent (re-check needed) — note: src/cleancad_core.py already implements FreeCAD-API-based face-adjacency traversal (groups()), usable as a starting point | FR-FEAT-02, NFR-PERF-02 (W), NFR-REL-03 (W) |
| M3 — Feature Recognition | Classify each recognized feature (hole/fillet/chamfer/rib/boss/emboss); estimate impact on defeaturing accuracy. | GNN model — team's choice (UV-Net/BRepNet/AAGNet/BrepMFR), must run on CPU at inference — mentor-suggested candidate (unverified, open): logos (characters, markings, embossed/engraved text) as an additional feature category | FR-FEAT-01, FR-FEAT-03, FR-FEAT-04 (W), FR-FEAT-08 (W), NFR-PERF-02 (W), NFR-PERF-03, NFR-REL-03 (W), NFR-ACC-01 |
| M4 — Decision Logic | Stage 1: rule-based (by size). Stage 2: ML-based. Proposes a keep/remove decision per feature — never auto-finalized. Wrong cases are corrected manually and fed back into Stage 2 training. | Rule engine + GNN classifier — mentor-suggested candidates (unverified, open): explicit dimension thresholds (hole diameter, chamfer size, fillet radius) for Stage 1; a supervisory decision layer to resolve rule/ML conflicts | FR-FEAT-03, FR-FEAT-06, FR-FEAT-07 (W), FR-FEAT-09 (W), NFR-PERF-02 (W), NFR-REL-03 (W), NFR-ACC-01 |
| M5 — Removal & Healing | Remove features approved in M4's proposal; run geometry healing so the result is a valid, watertight solid. On failure, store the geometry + failed face ID for manual correction. Mentor-suggested (unverified, open): track removal-execution failures as a separate category from wrong feature selection. | FreeCAD Python API, headless | FR-DEF-01, FR-DEF-02, FR-DEF-03, FR-DEF-04, FR-DEF-05 (W), NFR-PERF-02 (W), NFR-REL-02, NFR-REL-03 (W), NFR-REL-04, NFR-PORT-03 |
| M6 — Output & Reporting | Export the simplified STEP file; generate a visual (image-based) before/after comparison report. | Same FreeCAD toolchain + rendering | FR-IO-03, FR-VAL-01, FR-VAL-02, FR-VAL-06, NFR-USE-01 |
| M7 — Engineer Review | Present the visual comparison report to the engineer; the engineer accepts or rejects the result. Mandatory for every run. | TBD (interface format not yet decided) | FR-FEAT-05, NFR-USE-03 |
| M8 — Validation | Evaluate mesh quality (element count, generation time) and allow visual inspection. Success = defeaturing accuracy, not downstream CAE analysis accuracy (out of scope). | Gmsh (mesh quality), FreeCAD (visual check) | FR-DEF-05 (W), FR-VAL-03, FR-VAL-04 (W), FR-VAL-05, FR-VAL-07 (W), FR-DATA-03, FR-DATA-04 (W), NFR-ACC-02, NFR-ACC-03 (W) |

# 4. Interfaces Between Modules

| From -> To | Data Passed |
| --- | --- |
| M1 -> M2 | In-memory B-rep model (FreeCAD document/shape object) |
| M2 -> M3 | Graph structure: nodes = faces (with UV-sampled geometric attributes), edges = face adjacency |
| M3 -> M4 | Per-feature classification label + confidence score |
| M4 -> M5 | Proposed list of features to remove |
| M5 -> M6 | Modified, healed B-rep model |
| M6 -> M7 | Visual comparison report (rendered before/after images) + candidate output STEP file |
| M6 -> M8 | Output STEP file (original + defeatured) for mesh quality evaluation |
| M7 -> Output | Approve: finalize and release the STEP file. Reject: discard or route back for manual handling (policy TBD). |
| M5 (failure) -> Training Data | Geometry + failed face ID stored, manually corrected, fed into the retraining loop |
| Training: datasets -> M2 | Public dataset CAD files + self-authored simple CAD shapes + manually-corrected failure cases |
| Training: M2 -> Model Training -> M3 | Graph-converted training data -> trained GNN weights -> deployed into M3 |

# 5. Technology Stack Summary

| Layer | Choice | Status |
| --- | --- | --- |
| STEP I/O & geometry manipulation | FreeCAD Python API, headless/standalone (cadquery optional, STEP-generation only) | Confirmed 2026-09-18 |
| Visual/mesh validation | FreeCAD, Gmsh | Confirmed |
| B-rep -> graph conversion | occwl or FreeCAD-API equivalent | Needs re-check (occwl is built on pythonocc-core) |
| Feature recognition model | UV-Net / BRepNet / AAGNet / BrepMFR | Team's own choice — no mentor preference; must support CPU inference |
| Input/output CAD format | STEP only | Confirmed — NX out of scope |
| Training/validation data | Public datasets + self-authored shapes + failure-case retraining | Confirmed — no HP data |
| Inference deployment | GPU-less office laptop | Confirmed 2026-09-18 — training may use a GPU server |

See docs/step_library_comparison.md for the detailed comparison, mentor Q&A, and rationale behind each choice.

# 6. Design Constraints

- STEP is the only supported CAD format; NX support is explicitly out of scope. [FR-IO-04, NFR-PORT-02]

- The system must not store or transmit any confidential HP CAD data. [NFR-SEC-01, NFR-SEC-02]

- Engineer review (M7) is mandatory for every run, not conditional on model confidence. [FR-FEAT-05, NFR-USE-03]

- The comparison report (M6 output) must be visual; a text-only summary does not satisfy the requirement. [FR-VAL-01, FR-VAL-02, NFR-USE-01]

- Every output CAD model must be a valid, watertight solid. [FR-DEF-02, NFR-REL-04]

- Geometry processing (inference) must run without a GPU; only training may use a GPU server. [NFR-PERF-03]

- Success is measured by defeaturing accuracy; downstream CAE analysis accuracy (stress/deformation) is explicitly out of scope. [FR-VAL-05, NFR-ACC-01, NFR-ACC-02]

# 7. Requirements Traceability

Each functional (FR) and non-functional (NFR) requirement from the FRD/NFRD is mapped to the module(s) that realize it. "Training" means the offline model training pipeline; "All" means the requirement applies system-wide. (W) = Want priority (candidate, not yet committed); all others are Must.

| Requirement ID | Requirement | Module(s) | Priority |
| --- | --- | --- | --- |
| FR-IO-01 | STEP file input | M1 | Must |
| FR-IO-02 | Parse STEP to B-rep (FreeCAD, headless) | M1 | Must |
| FR-IO-03 | Export simplified STEP | M1, M6 | Must |
| FR-IO-04 | STEP only; NX out of scope | M1 | Must |
| FR-FEAT-01 | Identify shape features | M3 | Must |
| FR-FEAT-02 | B-rep to face-adjacency graph | M2 | Must |
| FR-FEAT-03 | Classify safe-to-remove features | M3, M4 | Must |
| FR-FEAT-04 | Confidence score per classification | M3 | Want |
| FR-FEAT-05 | Engineer review of all decisions | M7 | Must |
| FR-FEAT-06 | Two-stage decision logic (rule, then ML) | M4 | Must |
| FR-FEAT-07 | Explicit Stage 1 dimension thresholds | M4 | Want |
| FR-FEAT-08 | Logo feature category | M3 | Want |
| FR-FEAT-09 | Supervisory layer for rule/ML conflicts | M4 | Want |
| FR-DEF-01 | Remove features classified safe | M5 | Must |
| FR-DEF-02 | Geometry healing to watertight model | M5 | Must |
| FR-DEF-03 | Preserve external dimensions/topology | M5 | Must |
| FR-DEF-04 | Healing failure: skip, store geometry + face ID | M5, Training | Must |
| FR-DEF-05 | Removal-execution failures as separate category | M5, M8 | Want |
| FR-VAL-01 | Original vs. defeatured comparison report | M6 | Must |
| FR-VAL-02 | Visual (before/after) report | M6 | Must |
| FR-VAL-03 | Mesh quality / time-improvement evaluation | M8 | Must |
| FR-VAL-04 | Visual inspection in FreeCAD | M8 | Want |
| FR-VAL-05 | Defeaturing accuracy as primary criterion | M8 | Must |
| FR-VAL-06 | Red-marked removal faces saved as STEP | M6 | Must |
| FR-VAL-07 | Selection vs. execution accuracy reporting | M8 | Want |
| FR-DATA-01 | Train on public CAD datasets | Training | Must |
| FR-DATA-02 | Self-authored CAD shapes | Training | Must |
| FR-DATA-03 | Test-shape spec (thickness, width, 3 types) | Training, M8 | Must |
| FR-DATA-04 | Expanded validation dataset | Training, M8 | Want |
| NFR-PERF-01 | 70-90% preprocessing time reduction | All | Must |
| NFR-PERF-02 | Reasonable time per part | M2, M3, M4, M5 | Want |
| NFR-PERF-03 | CPU-only inference | M3 | Must |
| NFR-REL-01 | Original input file unchanged | M1 | Must |
| NFR-REL-02 | Healing failure must not crash / corrupt output | M5 | Must |
| NFR-REL-03 | Moderately complex geometry | M1, M2, M3, M4, M5 | Want |
| NFR-REL-04 | Valid, watertight output solid | M5 | Must |
| NFR-PORT-01 | Desktop OSes, no licensed CAD | All | Must |
| NFR-PORT-02 | ISO 10303 STEP conformance | M1 | Must |
| NFR-PORT-03 | Headless, standalone batch execution | M1, M5 | Must |
| NFR-SEC-01 | No confidential HP data in repository | All | Must |
| NFR-SEC-02 | Permitted training/validation data sources | Training | Must |
| NFR-USE-01 | Report readable by non-AI expert | M6 | Must |
| NFR-USE-02 | Clear error messages | M1 | Must |
| NFR-USE-03 | Decision-support, not autonomous | M7 | Must |
| NFR-MAINT-01 | Modular codebase per pipeline layer | All | Must |
| NFR-MAINT-02 | Git version control | All | Must |
| NFR-ACC-01 | Optimize defeaturing accuracy | M3, M4 | Must |
| NFR-ACC-02 | CAE analysis accuracy not measured | M8 | Must |
| NFR-ACC-03 | Selection vs. execution accuracy definition | M8 | Want |

# Related Documents

- docs/architecture.md (source diagrams and open points)

- docs/step_library_comparison.md (tech stack rationale and mentor Q&A)

- docs/Functional_Requirements_Document.docx / docs/Non_Functional_Requirements_Document.docx